DR-16.4

PC ControlEditor

User's Manual

Preparation

1

1.1

In order to use the control software to control the machine, we need to prepare the following items

(1)DR-16.4 Device

(2)PC with windows 7 or windows 10 and installed Net Framework 4.5 or higher version

(3)A wireless router

(4)One network cable or two

1.2

Use a network cable to connect the DR-16.4 to a wireless router and PC connect to the wireless router by WIFI or network cable (if you have another one)

2

Link And Control

Now launch the Control Editor.

You need to select  the network interface and connect the device before you use other features.

Click the search button witch in the upper left corner of the window.

The Device List will jump out and start to search device. And It will automatically connects to the device when it found one. If it doesn't do this. You can click the Research button witch under the device list or close it and try again later.

If the LED go green. It's mean connect successfully.

3

Features

3.1 The Faders panel

You can see the two panels at the bottom of the windowas below pictures show.

In this panel you can control all the input and output channels’ level, solo, mute, pan and rename the channel.

3.2 The DSP panel

In the DSP panel you can control single channel's Gate, COMP and PEQ

3.2.1 GATE

The Noise gate attenuates signals that below the threshold and allows signals to pass through only when they are above a threshold setting.

3.2.2 COMP

A compressor reduces the level of an audio signal if its amplitude exceeds a certain threshold.

3.2.3 PEQ

An equalizer is a filter that allows you to adjust the level of frequency in the range of 20Hz-20KHz.

3.3 Routing

In this panel, user can select one or several channels to assign the signal to corresponding outputs.

Output channel select

Input channel member  select

.Output channel select

You can select the output channel to change setting

.Input channel member select

Output channel setting

3.4 GEQ

In this panel, you can control single output GEQ

Output channel select

GEQ setting

Default

Disable GEQ

.Output channel select

Select to control

.GEQ setting

set the 31-band EQ

3.5 Auto Mix

In this panel, you can select several input channel as sources and active Auto Mix

3.6 Digital IO panel

You can change the match Digital IO setting in this panel.

3.7 FBC Setting

In this panel, you can set the Local input/output matching, FBC level and the EQ.

3.8 Ducker Setting

In this panel, you can set the Ducker local input , BGM source and Ducker Setting.

3.9 FX Setting

In this panel, you can change FX Effect setting

select Effect

Set Current Value

3.10 Save&Load and Copy

In S&L panel you can save and load 4 kinds of preset to PC or T2208.

Or delete them.

Select preset type

c

Select PC or Device

Operate button

In Copy panel you can select a channel as source and copy to other channel.

Source select

Target select

Property select

3.11 System

In the System panel you can set the device name